General Risk Assessment Form

| Date: (1) | Assessed by: (2) | Checked / Validated* by: (3) | Location: (4) | Assessment ref no (5) | Review date: (6) |
| --- | --- | --- | --- | --- | --- |
| Task / premises: (7) Handling of Sharps (including scalpels and hypodermic needles and broken glass) |  |  |  |  |  |

| Activity (8) | Hazard (9) | Who might be harmed and how (10) | Existing measures to control risk (11) | Risk rating (12) | Result (13) |
| --- | --- | --- | --- | --- | --- |
| Disposing of sharps | Sharp blades, needle and pins | User, risk of cuts, lacerations and needle stick injury. Risk of exposure to hazardous materials. All other staff, and students, cleaning staff and contractors - cuts, lacerations and needle stick injury risk of exposure to hazardous materials if waste is disposed of incorrectly or due to careless users. | Always cut away from yourself. Those under 18 must be supervised at all times when using scalpels blades. Sharps should never be left unattended on benches, never left on the edges of benches where they may fall and when the sharp is put down the blade / needle points away from the user and those in the vicinity. Sharps should NEVER carried in pockets, not kept loose in drawers / on benches and are stored either in the original packet or a plastic box with a lid. When a blade or needle is dropped it is picked up it must be picked up immediately with tweezers or blunt forceps - not unprotected hand. If a sharp is noted on the floor it’s not ignored but picked up with tweezers and disposed of in a sharps bin. Used blades and needles are disposed of in a designated yellow sharps bin (clinical waste bin). The bin must not be filled to beyond the appropriate point. Items are not left to stick out of the tip of a bin. As soon as bins are full the lids are firmly secured, put in a yellow bin for incineration (clinical waste bin on 3rd or basement floor). | Low | A |
| Changing / removing and fitting a scalpel blade | Sharp blade | User, cuts, lacerations and exposure to hazardous materials. All other staff, and students, cleaning staff and contractors - cuts, lacerations and needle stick injury risk of exposure to hazardous materials if waste is disposed of incorrectly or due to carless users | Users are warned not to get distracted from the task and others in the vicinity will not distract them. The following "lift and push" technique is used: - Grasp the handle of the scalpel with one hand and the round point forceps with the other hand. - Grip the part of the blade held down by the safety tab with the forceps and very carefully pry the blade up, sliding it off the safety tab and handle. - Dispose the used blade in a sharps bin. - Peel the ends of the package holding the replacement blade apart at the end that slides onto the handle; do not remove the blade for the packaging from the blade end. - Hold the blade end with the forceps and carefully slide the blade onto the handle until it snaps in place. | Low | A |
| Changing / use / removing / fitting a needle. | Needle | User, needle stick injury and exposure to hazardous materials. All other staff, and students, cleaning staff and contractors - cuts, lacerations and needle stick injury risk of exposure to hazardous materials if waste is disposed of incorrectly or due to carless users | Hypodermic needles are not be used where a safer alternative is available. Needles are fitted to the syringe while still sheathed, and are not re-sheathed. When un-sheathing needles the following is done: rotate needle within protective case (using packaging), to release needle from case safely. Needles are disposed of in a sharps bin as soon as possible. Never pass unsheathed needles around. If a hypodermic is dropped then it is immediately picked up using blunt nose tweezers and disposed of in a sharps bin. First aiders available throughout the building, call Security on 69966 out of hours for first aid treatment. First aid boxes available close to all main labs. If a needle stick injury occurs, encourage the wound to gently bleed, ideally holding it under running water, dry the wound and cover it with a waterproof plaster or dressing. Report the injury. | Low | A |
| Disposing of broken glass | Sharp objects and potentially hazardous materials | User, all others in vicinity. Cuts, lacerations, eye injuries and potential exposure to hazardous material All other staff, and students, cleaning staff and contractors - cuts, lacerations Media prep staff – cuts | Broken glass is cleaned using a dust pan and brush and safety spectacles are worn if there are large pieces to pick up; cut resistant gloves are worn. Users know that special care is needed when clearing broken glass from a sink where water can make sharp edges invisible: tongs or round nose forceps are used to pick out pieces. Broken glass is disposed of in a yellow sharps bin (clinical waste bin). | Low | A |

Authorisation by PI or Line Manager (15) I confirm that I have considered and understand the experiment and the associated hazards. I am satisfied that all of the hazards have been identified and that the control measures to be followed will reduce the risks to as low as is reasonably practicable.

Print name: Signed: Date:

Validated by:  (17)

Signature:

Date:

Authorisation by PI / line manager I confirm that I have considered and understand the risks and agree to abide by the control measures My counter-signature indicates that I am happy for the work to proceed and confirms my staff and students will be appropriately trained. Print name: Signed: Date:

| Name (please print) | signed | PI countersignature | date |
| --- | --- | --- | --- |