General Risk Assessment Form

| Date: (1) | Assessed by: (2) | Checked / Validated* by: (3) | Location: (4) | Assessment ref no (5) | Review date: (6) One year from issue or if significant changes |
| --- | --- | --- | --- | --- | --- |
| Task / premises: (7) Using Portable Power Tools |  |  |  |  |  |

| Activity (8) | Hazard (9) | Who might be harmed and how (10) | Existing measures to control risk (11) | Risk rating (12) | Result (13) |
| --- | --- | --- | --- | --- | --- |
|  | Electricity | Users, Electric shock | Check condition of lead and plug before use. Use 110v or battery tools or RCD where practicable. Check for hidden/buried cables before drilling etc. Do not work where water is present without specialist advice. Qualified person to test all portable electrical hand tools at least annually. | Low | A |
| Using Portable Power Tools | Flammable/explosive atmosphere | User and others in the vicinity, Fire/Explosion | Do not work near flammables, compressed gases, in explosive atmospheres or confined spaces without specialist advice. Check with line manager/supervisor before using flammable fuel or gas driven equipment. | Medium | A |
|  | Moving parts | User, Entanglement | Loose clothing, jewellery (removed) and long hair (tied back) to be kept clear of moving parts. Use guards where appropriate. | Low | A |
|  | Flying debris, swarf etc. | User and others in the vicinity, Eye, hand or facial injury | Use protective eyewear or face shield. Use guards where appropriate. Wear protective gloves where appropriate. Advise nearby persons of hazard. Isolate area with barriers, tape etc. where necessary | Low | A |
|  | Noise | User and others in the vicinity, Hearing damage | Check manufacturers noise value information. Consider frequency and duration, using the HSE noise exposure calculator to determine if under action/limit value. Wear hearing protection if above 80dB(A) or if uncomfortably loud (request assessment if in doubt). Advise nearby persons of hazard. Supervisors should inform users of risks from noise. | Low | A |
|  | Vibration | User, Hand/Arm Vibration Syndrome (HAVS) Carpal Tunnel Syndrome | Select power tools with lowest vibration levels. Minimise the time and frequency individuals use the equipment (e.g. job rotation). Restrict use of vibration inducing tools to recommended times (see Departmental guidance, manufacturer’s information, local risk assessment or label on equipment/ box). Use HSE hand-arm vibration exposure calculator to determine acceptable usage. Ensure tools are properly stored, maintained and used according to manufacturer’s instructions. Supervisors should inform users of risks from vibration. Arrange health surveillance for those identified at risk from vibration. | Low | A |
|  | Ergonomic | User, Musculo-skeletal injury | Ensure there is adequate room to do the job. Minimise the time and frequency individuals use heavy equipment (e.g. job rotation). Use jigs and suspension systems to assist the handling of heavy equipment (e.g. large grinders). | Low | A |
|  | Slips, trips and falls | User and others in the vicinity, falling over, impact from floor, other furniture. | Ensure work area is free (as is practicable) from trailing cables, tools, materials, debris and spills. All work should be from a suitable and stable work platform. | Low | A |
|  | Dust | User and others in the vicinity, Respiratory illness Reduced visibility Skin irritation | Use local exhaust ventilation where possible. Increase ventilation to work area (e.g. open windows, temporary extract etc.). Use positive pressure RPE if required, Speak to safety advisor for more information. CRA form for hazardous materials. Wear close-fitting safety goggles. Stop working if visibility is noticeably reduced. Wear suitable protective clothing. Clean work area regularly, vacuum dust do not sweep as this will increase the risk of airborne particulates, wash down surfaces and hands after use). | Medium | A |
|  | Air line detaching or bursting | User and others in the vicinity, Eye/face injury Distraction | Check condition and connections of air-line before use. Ensure air-line is not on a traffic route (pedestrian or vehicular). | Low | A |
|  | Tool jamming or binding | User, Wrist/hand injury | Check tool is appropriate for the job and used in accordance with manufacturer’s instructions. Trained operators only (or under strict supervision). Ensure tools are maintained according to manufacturer’s instructions. | Low | A |
|  | Inappropriate use | User and others in the vicinity, | Check tool is appropriate for the job and used in accordance with manufacturer’s instructions. Users shall be trained in the correct use of portable tools. Inexperienced power tools users should be supervised or observed when first using an unfamiliar item of equipment or in an unfamiliar environment. Power tools should be securely stored when not in use. | Medium | A |
|  | Lone working | User, no emergency response | No lone working should occur at any time when using power tools. See P&A H&S policy and guidance on lone working. | Low | A |

| Action plan (14) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Ref No | Further action required | Action by whom | Action by when | Done |

Authorisation by Supervisor / PI / Line Manager (15) I confirm that I have considered and understand the procedure and the associated hazards. I am satisfied that all of the hazards have been identified and that the control measures to be followed will reduce the risks to as low as is reasonably practicable.

Print name: Signed: Date:

Validated by:  (16) 
Signature: 

Date:

Declaration by user and supervisor / PI / Line Manager (17) I confirm that I have read this Risk Assessment and that I understand the hazards and risks involved and will follow all of the safety procedures stated. I confirm that the user who has signed below is competent to undertake the work

| Name (please print) | Role | Signed | Date |
| --- | --- | --- | --- |